COMPUTER ORGANIZATION AND ASSEMBLY LANGUAGE

Lab 7

BASIC ARITHMETIC OPERATION

Name: M.Noman javed

Roll#: 2024F-BCS-102

SECTION: F

TASK# 1

| ADDRESS | DATA/CODE | MENEMONICS | COMMENTS |
| --- | --- | --- | --- |
| 0700:0100 | B8 11 11 | MOV AX, 1111 | The value of 1111h is copied to AX |
| 0700:0103 | BB 23 23 | MOV BX, 2323 | The value of 2323h is copied to BX |
| 0700:106 | B9 45 30 | MOV CX, 3045 | The value of 3045h is copied to CX |
| 0700:109 | BA 41 42 | MOV DX, 4241 | The value of 4241h is copied to AX |
| 0700:10C | 03 C1 | ADD AX, CX | AX is CX added and saved at AX. Carry flag will be 0. |
| 0700:10E | 13 DA | ADC BX, DX | Apply carry of AX and CX addition to BX and DX addition and saved at BX |

TASK# 2

| ADDRESS | DATA/CODE | MENEMONICS | COMMENTS |
| --- | --- | --- | --- |
| 0700:0100 | B8 00 30 | MOV AX, 3000 | Loads 3000h into AX. |
| 0700:0103 | BB 00 20 | MOV BX, 2000 | Loads 2000h into BX. |
| 0700:106 | B9 00 10 | MOV CX, 1000 | Loads 1000h into CX. |
| 0700:109 | BA 00 40 | MOV DX, 4000 | Loads 4000h into DX. (Your original comment incorrectly said AX.) |
| 0700:10C | 2B C1 | SUB AX, CX | Performs AX = AX − CX = 3000h − 1000h = 2000h. No borrow → CF = 0. |
| 0700:10E | 1B DA | SBB BX, DX | Performs BX = BX − DX − CF = 2000h − 4000h − 0 = E000h. Uses carry flag as borrow. |

TASK# 3

| ADDRESS | DATA/CODE | MENEMONICS | COMMENTS |
| --- | --- | --- | --- |
| 0700:0100 | B3 05 | MOV BL, 05H | Loads 05h into BL.. |
| 0700:0102 | B1 0A | MOV CL, 10H | Loads 0Ah into CL |
| 0700:104 | 8A C1 | MOV AL, CL | Copies CL To AL (AL becomes 0Ah) |
| 0700:106 | F6 E3 | MUL BL | Unsigned multiply: AX = AL × BL = 0Ah × 05h = 32d = 20h. |
| 0700:108 | 8B D0 | MOV DX, AX | Copies AX To DX (DX becomes 20h). |

TASK# 4

| ADDRESS | DATA/CODE | MENEMONICS | COMMENTS |
| --- | --- | --- | --- |
| 0700:0100 | B3 05 | MOV BL, 05H | Loads 05H into BL.. |
| 0700:0102 | B1 10 | MOV CL, 10H | Loads 0Ah into CL |
| 0700:104 | B4 00 | MOV AH, 00 | Loads 00H into AH. |
| 0700:106 | 8A C1 | MOV AL, CL | Copies CL → AL (AL becomes 10H). |
| 0700:108 | F6 F3 | DIV BL | Performs AL = AL / BL, AH = AL mod BL. Since AL = 10H, BL = 05H To quotient = 03H, remainder = 00H. Result: AL = 03H, AH = 00H. |
| 0700:10A | 8B D0 | MOV DX, AX | Copies AX To DX (DX becomes 0003H). |